PYTHON CODE

import speech_recognition as sr

#import serial

import time

# Adjust the port to match your system

#arduino = serial.Serial('COM3', 9600, timeout=1)

time.sleep(2)  # Wait for Arduino to reset

recognizer = sr.Recognizer()

mic = sr.Microphone()

print("Say 'open ' or 'close '...")

while True:

with mic as source:

print("Listening...")

audio = recognizer.listen(source)

try:

command = recognizer.recognize_google(audio).lower()

print("You said:", command)

if "open" in command:

#arduino.write(b"open\n")

print("Sent: open")

elif "close" in command:

#arduino.write(b"close\n")

print("Sent: close")

else:

print("Command not recognized")

except sr.UnknownValueError:

print("Could not understand audio")

except sr.RequestError as e:

print(f"Error with speech recognition service: {e}")

ESP32 CODE

//Header File

#include<Wire.h>

#include <LiquidCrystal.h>

LiquidCrystal lcd(18,5,17,16,4,0);

// Include the Servo library

#include <Servo.h>

int serialA;

volatile int i=0;//initializing a integer for incrementing and decrementing duty ratio.

// Declare the Servo pin

int servoPin1 = D5;

// Create a servo object

Servo Servo1;

int incomingByte =0;

int buz = D0;

void setup() {

// We need to attach the servo to the used pin number

pinMode(buz, INPUT);

Servo1.attach(servoPin1);

Serial.begin(9600);

Serial.print('System Ready');

lcd.begin(16,2);

lcd.clear();

lcd.print("VOICE BASED DOOR");

lcd.setCursor(0,1);

lcd.print("OPEN/CLOSE SYSTEM");

digitalWrite(buz, HIGH);

delay(1000);

lcd.clear();

digitalWrite(buz, LOW);

}

void loop(){

if (Serial.available() > 0)

{

incomingByte = Serial.read();

}

switch(incomingByte)

{

case '1':

// Make servo go to 0 degrees

delay(100);

lcd.setCursor(0,0);

lcd.print("STATUS: UNLOCKED  ");

lcd.setCursor(0,1);

lcd.print("  SUCCESSFULLY   ");

Servo1.write(0);

delay(2000);

incomingByte='*';

break;

case '2':

// Make servo go to 0 degrees

Servo1.write(180);

lcd.setCursor(0,0);

lcd.print("STATUS: LOCKED   ");

lcd.setCursor(0,1);

lcd.print("  SUCCESSFULLY     ");

//delay(2000);

incomingByte='*';

break;

delay(1000);

}

}